Raspberry Pi – interactive music instrument

Welcome to the Raspberry Pi booth!

In this demo, you will perform a few simple tasks to make the device responsive to your hand gestures and play music.

- Make sure that the raspberry pi is turned off and connect the sensor to the Raspberry Pi with the 4 connections wire:

Gnd -> Ground

Echo -> GPIO_xx (from step 2)

Trig -> GPIO_xx (from step 2)

Vcc -> 5v

- Connect the power wire to the RP (Raspberry PI).

- Once your Raspberry Pi has completed the boot process, a login prompt will appear, login with User: pi password: raspberry

- Type “cd SAP-dokm-handson/RP” and press enter.

- Type “startx” in order to start up the desktop (this may take a few seconds).

- Open file manager

- Open the file “exercise.py” for edit using IDLE (right click the file).

- Set a value for GPIO_ECHO_BEEP and GPIO_TRIG_BEEP (one of the GPIO pins of the Raspberry pi) – step 1 in the code.

- Set the distance by calling the function                                “distance(GPIO_ECHO_BEEP, GPIO_TRIG_BEEP)” and assign it to the parameter “measured_distance” -  (step 2 in the code)

- Set the desire distance threshold that should start the playing (Step 3 in the code)

- Ctrl+s to save the file.

- Run the code:

  - Double click on “LXTerminal”.

  - Write: “cdrp” and press enter.

  - Write “sudo  ./exercise.py” and press enter.

  - Password: “admin”.

- Once you hear the background music, put your hand above the distance sensor to start playing along.